SNB Series

Overview

The SNB series capacitors with double side metallized

carrier film with internal series connection and metallized

polypropylene film, solvent resistant plastic case with

resin sealing (UL 94 V-0)

Application:

The SNB series is designed for snubber/pulse applications.

For high pulse and high frequency applications requiring

extremely reliable contacts e.g. IGBT-applications

Benefits

Self-healing

Very low dissipation factor

Very high ripple current

Internal series connection

Negative capacitance change versus temperature

Construction

Dielectric: Polypropylene (PP) film.

Capacitor electrodes: Double-sided metallized plastic film.

Protection: Solvent resistant plastic case with resin sealing (UL 94 V-0)

Terminals: Tinned copper lugs (lead-free)

Internal construction:

Structure of ordering code

SNB - 1200 – 0.47 - T25  0

1     2   3    4   5

1  Series code

2  Rated voltage: 1200 VDC

3  Rated capacitance: 0.47 μF

4  Designs Type

5  Internal use

SNB Series

Electrical data

Reference standards                         IEC 61071 , IEC 60068 , RoHS compliance.

Degree of protection                         IP00

Rated capacitance (CN)                       0.047μF ... 7.5μF

Rated voltage (UNDC)                         700V ... 3000V

Standard capacitance tolerance                 K: ±10% , J: ±5%

Dissipation factor tan δ (1kHz@20°C)            ≤ 5.0 • 10-4

Test voltage between terminals UTT              1.5 UNDC , 10s

Test voltage between terminals and case UTC      4000 VAC, 10s

Insulation Resistance                      30000s but need not exceed 30GW (typical value), after

1 minute of electrification at 100Vdc (25 ± 5°C).

Self inductance                           1nH/mm of fixing pitch

Operating temperature range (case)            –40 °C ... +105°C

Max. permissible ambient temperature          +85°C, operation at rated power, rated current and

natural cooling

Storage temperature Θstg                    –40 ... +105 °C

Climatic category                          40/105/56

Capacitance deviation                      in the operating Temperature range of -40 to +85 °C,

±1.5% max on capacitance value measured at +20 °C

Damp heat test      - Test conditions          Temperature : +40 °C

Relative humidity : 93% ±2%

Test duration : 56 days

- Performances           Capacitance change : ≤ ± 5%

tgδ change: ≤50% of nominal value at 1 kHz

Insulation resistance: ≤50% of limit value

Expected lifetime                            100 000 h at UNDC @ Θhs 85°C

30 000 h at URMS @ Θhs 85°C

Capacitance drop at end of life                 -3% (typical)

Fit rate                                    50 (100 000 h at Θhs 85°C)

Installation                                Whatever position assuring correct heat dissipation. Arrangement of many components with box walls in contact not admitted; suggested minimum distance between side by side elements ≥1/8 of the box thickness.

SNB Series

UN  3000V      Us  4500V      Urms  1000V

| Ordering code | Cn (μF) | DIMENSIONS (mm) |  |  | du/dt (V/μs) | Ipk (A) | ESR(2) (mΩ) | Imax(1) (A) | Designs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | L | B | H |  |  |  |  |  |
| SNB-3000-0.10-T220 | 0.10 | 42.0 | 17 | 28 | 1780 | 178 | 11.6 | 8 | T22 |
| SNB-3000-0.10-T250 | 0.10 | 42.0 | 17 | 28 | 1780 | 178 | 11.6 | 8 | T25 |
| SNB-3000-0.10-U220 | 0.10 | 42.0 | 17 | 28 | 1780 | 178 | 11.6 | 8 | U22 |
| SNB-3000-0.10-U250 | 0.10 | 42.0 | 17 | 28 | 1780 | 178 | 11.6 | 8 | U25 |
| SNB-3000-0.15-T220 | 0.15 | 42.0 | 20 | 39 | 1780 | 267 | 8.7 | 10 | T22 |
| SNB-3000-0.15-T250 | 0.15 | 42.0 | 20 | 39 | 1780 | 267 | 8.7 | 10 | T25 |
| SNB-3000-0.15-U220 | 0.15 | 42.0 | 20 | 39 | 1780 | 267 | 8.7 | 10 | U22 |
| SNB-3000-0.15-U250 | 0.15 | 42.0 | 20 | 39 | 1780 | 267 | 8.7 | 10 | U25 |
| SNB-3000-0.22-T220 | 0.22 | 42.0 | 28 | 37 | 1780 | 392 | 5.3 | 13 | T22 |
| SNB-3000-0.22-T250 | 0.22 | 42.0 | 28 | 37 | 1780 | 392 | 5.3 | 13 | T25 |
| SNB-3000-0.22-U220 | 0.22 | 42.0 | 28 | 37 | 1780 | 392 | 5.3 | 13 | U22 |
| SNB-3000-0.22-U250 | 0.22 | 42.0 | 28 | 37 | 1780 | 392 | 5.3 | 13 | U25 |
| SNB-3000-0.33-T250 | 0.33 | 57.0 | 30 | 45 | 1160 | 383 | 7.4 | 15 | T25 |
| SNB-3000-0.33-T390 | 0.33 | 57.0 | 30 | 45 | 1160 | 383 | 7.4 | 15 | T39 |
| SNB-3000-0.33-U250 | 0.33 | 57.0 | 30 | 45 | 1160 | 383 | 7.4 | 15 | U25 |
| SNB-3000-0.33-U390 | 0.33 | 57.0 | 30 | 45 | 1160 | 383 | 7.4 | 15 | U39 |
| SNB-3000-0.47-T250 | 0.47 | 57.0 | 30 | 45 | 1160 | 545 | 5.2 | 17 | T25 |
| SNB-3000-0.47-T390 | 0.47 | 57.0 | 30 | 45 | 1160 | 545 | 5.2 | 17 | T39 |
| SNB-3000-0.47-U250 | 0.47 | 57.0 | 30 | 45 | 1160 | 545 | 5.2 | 17 | U25 |
| SNB-3000-0.47-U390 | 0.47 | 57.0 | 30 | 45 | 1160 | 545 | 5.2 | 17 | U39 |
| SNB-3000-0.68-T250 | 0.68 | 57.0 | 35 | 50 | 1160 | 789 | 3.7 | 20 | T25 |
| SNB-3000-0.68T390 | 0.68 | 57.0 | 35 | 50 | 1160 | 789 | 3.7 | 20 | T39 |
| SNB-3000-0.68-U250 | 0.68 | 57.0 | 35 | 50 | 1160 | 789 | 3.7 | 20 | U25 |
| SNB-3000-0.68-U390 | 0.68 | 57.0 | 35 | 50 | 1160 | 789 | 3.7 | 20 | U39 |

(1) Maximum values at 100kHz, +60°C for case operating T= +85°C

(2) Typical values at 100kHz.